# Week 20 – Testing the AI model API.

Last week, I had issues hosting the AI model API on the Railway cloud server due to dependency issues from OpenCV. The version of OpenCV (opencv-python) I was using locally required some GUI components for compilation, which cannot be found on server environments. I researched and asked around until I came across a friend who suggested I use the opencv-contrib-python-headless. This package is smaller than the other opencv-python packages and is built for server (headless) environments. It also contains the main and contrib/extra modules my application needs; after installing this opencv version to my project, deploying the app on the railway server was successful.

Once the API was live, I decided to use the ESP 32 AI thinker module to take pictures and send them over the internet via the API for inference. To achieve the above objectives, I took the following steps.

- Use openssl to acquire a Root CA Certificate from my API domain to attach to the WifiClient on the ESP32 for HTTPS requests.

- Configure the ESP 32 camera to take pictures using the QVGA frame size for smaller file sizes.

- Encode the image taken from the ESP32 into base64 string and send the data as JSON to the API.

- Use the JsonSerialize Arduino library to encode the API payload into JSON format.

- Prep the API route to accept the base64 string and decode the string to an image for inference.

After taking the above steps, I could finally make a successful request to the API after numerous days of debugging and research.

The Image

The Inference

# Project Update

- Hosted the API model on the cloud for HTTPS access.

- I have configured ESP32 to request the AI trash classifier API successfully.

- 3D modelled and printed some additional parts of my prototype.

- I have started integrated circuit planning.

- I purchased perf boards and tools for the hardware implementation.

# Reflection

Hosting and configuring the AI model API has been challenging, but I am glad I solved the issue. I realise now how important it is to network and discuss the problems I may face in my projects, whether online or in person, with robotics enthusiasts. Usually, I come across a tip that would help me solve a bug I have been facing for a while. Moving forward, I will engage in similar discussions to help or get technical advice from the professionals.